- Preparation tasks (done before the lab at home):

  - Table with segments values for display 0 to 9 on a common anode 7-segment display,

| Digit | A | B | C | D | E | F | G | DP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 3 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 4 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 6 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 7 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 9 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |

  - In your words, describe the difference between Common Cathode and Common Anode 7-segment display.

7-segment display with common cathode has common cathode for LEDs of all 7 segments and decimal point. So any segment can be lit seting its anode pin to 0.

Common anod is oposite. Segments are lit with 1.

- 7-segment library. Submit:

  - Listing of library source file segment.c,

/***********************************************************************

*

* Seven-segment display library for AVR-GCC.

* ATmega328P (Arduino Uno), 16 MHz, AVR 8-bit Toolchain 3.6.2

*

* Copyright (c) 2019-2020 Tomas Fryza

* Dept. of Radio Electronics, Brno University of Technology, Czechia

* This work is licensed under the terms of the MIT license.

*

**********************************************************************/

/* Includes ----------------------------------------------------------*/

#define F_CPU 16000000

#include <util/delay.h>

#include "gpio.h"

#include "segment.h"

/* Variables ---------------------------------------------------------*/

// Active-low digits 0 to 9

uint8_t segment_value[] = {

// abcdefgDP

0b00000011,		// Digit 0

0b10011111,		// Digit 1

0b00100101,		// Digit 2

0b00001101,		// Digit 3

0b10011001,		// Digit 4

0b01001001,		// Digit 5

0b01000001,		// Digit 6

0b00011111,		// Digit 7

0b00000001,		// Digit 8

0b00001001};	// Digit 9

// Active-high position 0 to 3

uint8_t segment_position[] = {

// p3p2p1p0....

0b00010000,		// Position 0

0b00100000,		// Position 1

0b01000000,		// Position 2

0b10000000};	// Position 3

/* Function definitions ----------------------------------------------*/

void SEG_init(void)

{

/* Configuration of SSD signals */

GPIO_config_output(&DDRD, SEGMENT_LATCH);

GPIO_config_output(&DDRD, SEGMENT_CLK);

GPIO_config_output(&DDRB, SEGMENT_DATA);

}

/*--------------------------------------------------------------------*/

void SEG_update_shift_regs(uint8_t segments, uint8_t position)

{

uint8_t bit_number;

segments = segment_value[segments];     // 0, 1, ..., 9

position = segment_position[position];  // 0, 1, 2, 3

// Pull LATCH, CLK, and DATA low

GPIO_write_low(&PORTD, SEGMENT_LATCH);

GPIO_write_low(&PORTD, SEGMENT_CLK);

GPIO_write_low(&PORTB, SEGMENT_DATA);

// Wait 1 us

_delay_us(1);

// Loop through the 1st byte (segments)

// a b c d e f g DP (active low values)

for (bit_number = 0; bit_number < 8; bit_number++)

{

// Output DATA value (bit 0 of "segments")

if ((segments & 1) == 0)

{

GPIO_write_low(&PORTB, SEGMENT_DATA);

}

else

{

GPIO_write_high(&PORTB, SEGMENT_DATA);

}

// Wait 1 us

_delay_us(1);

// Pull CLK high

GPIO_write_high(&PORTD, SEGMENT_CLK);

// Wait 1 us

_delay_us(1);

// Pull CLK low

GPIO_write_low(&PORTD, SEGMENT_CLK);

// Shift "segments"

segments = segments >> 1;

}

// Loop through the 2nd byte (position)

// p3 p2 p1 p0 . . . . (active high values)

for (bit_number = 0; bit_number < 8; bit_number++)

{

// Output DATA value (bit 0 of "position")

if ((position % 2) == 0)

{

GPIO_write_low(&PORTB, SEGMENT_DATA);

}

else

{

GPIO_write_high(&PORTB, SEGMENT_DATA);

}

// Wait 1 us

_delay_us(1);

// Pull CLK high

GPIO_write_high(&PORTD, SEGMENT_CLK);

// Wait 1 us

_delay_us(1);

// Pull CLK low

GPIO_write_low(&PORTD, SEGMENT_CLK);

// Shift "position"

position = position >> 1;

}

// Pull LATCH high

GPIO_write_high(&PORTD, SEGMENT_LATCH);

// Wait 1 us

_delay_us(1);

}

/*--------------------------------------------------------------------*/

/* SEG_clear */

/*--------------------------------------------------------------------*/

/* SEG_clk_2us */

  - Listing of decimal counter application main.c (at least two-digit decimal counter, ie. from 00 to 59),

/***********************************************************************

*

* Decimal counter with 7-segment output.

* ATmega328P (Arduino Uno), 16 MHz, AVR 8-bit Toolchain 3.6.2

*

* Copyright (c) 2018-2020 Tomas Fryza

* Dept. of Radio Electronics, Brno University of Technology, Czechia

* This work is licensed under the terms of the MIT license.

*

**********************************************************************/

/* Includes ----------------------------------------------------------*/

#include <avr/io.h>         // AVR device-specific IO definitions

#include <avr/interrupt.h>  // Interrupts standard C library for AVR-GCC

#include "timer.h"          // Timer library for AVR-GCC

#include "segment.h"        // Seven-segment display library for AVR-GCC

uint8_t singles = 0;

uint8_t decimals = 0;

/* Function definitions ----------------------------------------------*/

/**

* Main function where the program execution begins. Display decimal

* counter values on SSD (Seven-segment display) when 16-bit

* Timer/Counter1 overflows.

*/

int main(void)

{

// Configure SSD signals

SEG_init();

/* Configure 16-bit Timer/Counter0

* Set prescaler and enable overflow interrupt */

TIM0_overflow_4ms();

TIM0_overflow_interrupt_enable();

/* Configure 16-bit Timer/Counter1

* Set prescaler and enable overflow interrupt */

TIM1_overflow_1s();

TIM1_overflow_interrupt_enable();

// Enables interrupts by setting the global interrupt mask

sei();

// Infinite loop

while (1)

{

/* Empty loop. All subsequent operations are performed exclusively

* inside interrupt service routines ISRs */

}

// Will never reach this

return 0;

}

/* Interrupt service routines ----------------------------------------*/

/**

* ISR starts when Timer/Counter0 overflows. Display value on SSD

*/

ISR(TIMER0_OVF_vect)

{

static uint8_t position = 0;

if (position == 0)

{

SEG_update_shift_regs(singles, 0);

position = 1;

}

else

{

SEG_update_shift_regs(decimals, 1);

position = 0;

}

}

/**

* ISR starts when Timer/Counter1 overflows. Increment decimal counter

*/

ISR(TIMER1_OVF_vect)

{

singles++;

if(singles > 9)

{

singles = 0;

decimals++;

if(decimals > 5)

{

decimals = 0;

}

}

}

  - Screenshot of SimulIDE circuit.

- Snake. Submit:

  - Look-up table with snake definition,

  - Listing of your snake cycling application main.c.

uint8_t segment_value[] = {
// abcdefgDP
0b00000011, // Digit 0
0b10011111, // Digit 1
0b00100101, // Digit 2
0b00001101, // Digit 3
0b10011001, // Digit 4
0b01001001, // Digit 5
0b01000001, // Digit 6
0b00011111, // Digit 7
0b00000001, // Digit 8
0b00001001}; // Digit 9

// Active-high position 0 to 3
uint8_t segment_position[] = {
// p3p2p1p0....
0b00010000, // Position 0
0b00100000, // Position 1
0b01000000, // Position 2
0b10000000}; // Position 3

pro snake potřebuju lookup tabulku a čítač

dodělat seg_clear a seg_clock 4us