LAB6 home assignment						Lukáš Kaleta

- Preparation tasks (done before the lab at home). Submit:

  - Table with LCD signals,

| LCD signal(s) | AVR pin(s) | Description |
| --- | --- | --- |
| RS | PB0 | Register selection signal. Selection between Instruction register (RS=0) and Data register (RS=1) |
| R/W | GND | Choosing between reading (R/W = 1) or writing (R/W = 0 or GND) |
| E | PB1 | On the falling edge of E signal data transfers ito the LCD |
| D[3:0] | - | Lower 4 data pins. Used only for 8bit communication |
| D[7:4] | PD[7:4] | Upper 4 data pins. Usable for both 4bit and 8bit communication |

| Function name | Function parameters | Description | Example |
| --- | --- | --- | --- |
| lcd_init | LCD_DISP_OFF LCD_DISP_ON LCD_DISP_ON_CURSOR LCD_DISP_ON_CURSOR_BLINK | display off display on, cursor off display on, cursor on disp on, cursor flashing | lcd_init(LCD_DISP_OFF); lcd_init(LCD_DISP_ON); lcd_init(LCD_DISP_ON_CURSOR); lcd_init(LCD_DISP_ON_CURSOR_BLINK); |
| lcd_clrscr | - | clear screen | lcd_clrscr(); |
| lcd_gotoxy | x, y | Set cursor to specified position. | lcd_gotoxy(1,0); |
| lcd_putc | c | Wtrite one symbol on current position | lcd_putc(a); |
| lcd_puts | “string“ | Wtrite given string on current position | lcd_putc(“DE_2“); |
| lcd_command | cmd | Send LCD controller instruction command | lcd_command(uint8_t cmd); |
| lcd_data | data | Send data byte to LCD controller | lcd_data(uint8_t data) |

  - ASCII values.

A = $41

B = $42

…

Z = $5A

a = $61

…

z = $7A

0 = $30

1 = $31

…

9 = $39

- HD44780 communication. Submit:

  - Picture of time signals between ATmega328P and HD44780 (LCD keypad shield) when transmitting data DE2.

- Stopwatch. Submit:

  - Listing of TIMER2_OVF_vect interrupt routine with complete stopwatch code (minutes:seconds.tenths) and square value computation,

/* Interrupt service routines ----------------------------------------*/

/**

* ISR starts when Timer/Counter2 overflows. Update the stopwatch on

* LCD display every sixth overflow, ie approximately every 100 ms

* (6 x 16 ms = 100 ms).

*/

ISR(TIMER2_OVF_vect)

{

static uint8_t number_of_overflows = 0;

static uint8_t tens = 0;        // Tenths of a second

static uint8_t secs = 0;        // Seconds

static uint8_t mins = 0;        // Minutes

static uint16_t secs_square = 0; // squared number of secs on position A

char lcd_string[] = "  ";      // String for converting numbers by itoa()

number_of_overflows++;

if (number_of_overflows >= 5)

{

// Do this every 6 x 16 ms = cca 100 ms

number_of_overflows = 0;

// WRITE YOUR CODE HERE

tens++;

if(tens > 9)

{

tens = 0;

secs++;

if(secs > 9)

{

if(secs > 59)

{

secs = 0;

lcd_gotoxy(5, 0);

lcd_puts("0");

mins++;

if(mins > 9)

{

if(mins > 59)

{

mins = 0;

lcd_gotoxy(2, 0);

lcd_puts("0");

}

// displaying > 9 minutes

itoa(mins, lcd_string, 10);

lcd_gotoxy(1, 0);

lcd_puts(lcd_string);

}

else

{

// displaying < 10 minutes

itoa(mins, lcd_string, 10);

lcd_gotoxy(2, 0);

lcd_puts(lcd_string);

}

// clearing square of secs

lcd_gotoxy(11, 0);

lcd_puts("    ");

}

// displaying > 9 seconds

itoa(secs, lcd_string, 10);

lcd_gotoxy(4, 0);

lcd_puts(lcd_string);

}

else

{

// displaying < 10 seconds

itoa(secs, lcd_string, 10);

lcd_gotoxy(5, 0);

lcd_puts(lcd_string);

}

// computing square of secs

secs_square = secs * secs;

itoa(secs_square, lcd_string, 10);

lcd_gotoxy(11, 0);

lcd_puts(lcd_string)

}

// displaying tenths of seconds

itoa(tens, lcd_string, 10);

lcd_gotoxy(7, 0);

lcd_puts(lcd_string);

}

}

  - Screenshot of SimulIDE circuit when "Power Circuit" is applied.

- Progress bar. Submit:

  - Listing of TIMER0_OVF_vect interrupt routine with a progress bar,

/*--------------------------------------------------------------------*/

/**

* ISR starts when Timer/Counter0 overflows. Update the progress bar on

* LCD display every 16 ms.

*/

ISR(TIMER0_OVF_vect)

{

static uint8_t number_of_overflows = 0;

static uint8_t symbol = 0;

static uint8_t position = 1;

number_of_overflows++;

if (number_of_overflows >= 6)

{

number_of_overflows = 0;

symbol++;

if (symbol > 5)					// one bar is full

{

symbol = 1;

position++;

if (position > 9)

{

position = 1;				// clear position

lcd_gotoxy(1, 1);			// clear all bars

lcd_puts("          ");

}

}

// display bar

lcd_gotoxy(position, 1);

lcd_putc(symbol);

}

}

  - Screenshot of SimulIDE circuit when "Power Circuit" is applied.