- Preparation tasks (done before the lab at home). Submit:

  - Table with voltage divider, calculated, and measured ADC values for all buttons.

| Push button | PC0[A0] voltage | ADC value (calculated) | ADC value (measured in simulation) |
| --- | --- | --- | --- |
| Right | 0 V | 0 | 0 |
| Up | 0.495 V | 101 | 101 |
| Down | 1.203 V | 246 | 245 |
| Left | 1.970 V | 403 | 402 |
| Select | 3.182 V | 651 | 650 |
| none | 5 V | 1023 | 1022 |

| Operation | Register(s) | Bit(s) | Description |
| --- | --- | --- | --- |
| Voltage reference | ADMUX | REFS1:0 | 01: AVcc voltage reference, 5V |
| Input channel | ADMUX | MUX3:0 | 0000: ADC0, 0001: ADC1, ... |
| ADC enable | ADCSRA | ADEN | 1: enables the ADC |
| Start conversion | ADCSRA | ADSC | 1: start conversion in single mode and start first conversion in free mode. |
| ADC interrupt enable | ADCSRA | ADIE | 1: interrupt is enabled … When the I-bit in SREG is set, „ADC completed“ interrupt is activated |
| ADC clock prescaler | ADCSRA | ADPS2:0 | 000: Division factor 2, 001: 2, 010: 4, ... (for us: 111) |
| ADC result | ADCH ADCL | ADCH7:0 ADCL7:0 | When an ADC conversion is complete, the result is found in these two registers. |

| Function name | Function parameters | Description | Example |
| --- | --- | --- | --- |
| uart_init | UART_BAUD_SELECT (9600, F_CPU) | Initialize UART to 8N1 and set baudrate to 9600 Bd | uart_init(UART_BAUD_SELECT(9600, F_CPU)); |
| uart_getc | - | get data from UART | uart_getc( |
| uart_putc | c | Send one symbol to UART | uart_putc(a); |
| uart_puts | “string“ | Send given string to UART | uart_putc(“DE_2“); |

- ADC. Submit:

  - Listing of ADC_vect interrupt routine with complete code for sending data to the LCD/UART and identification of the pressed button.

/* -------------------------------------------------------------------*/

/**

* ISR starts when ADC completes the conversion. Display value on LCD

* and send it to UART.

*/

ISR(ADC_vect)

{

uint16_t value = 0;

char lcd_string[4] = "    ";

// Copy ADC result to 16-bit variable

value = ADC;

// Print to LD in decimal

itoa(value, lcd_string, 10);    // Convert to string in decimal

lcd_gotoxy(8, 0);

lcd_puts("    ");

lcd_gotoxy(8, 0);

lcd_puts(lcd_string);

// Send to uart in decimal

if (value < 700)

{

uart_puts("ADC value in decimal:");

uart_puts(lcd_string);

uart_puts("\n");		// \n... next line , \r line before

}

// Print on lcd in hex

itoa(value, lcd_string, 16);

lcd_gotoxy(13, 0);

lcd_puts("    ");

lcd_gotoxy(13, 0);

lcd_puts(lcd_string);

// Odd Parity bit counter

if(value %2 == 0)

{

lcd_gotoxy(15,1);

lcd_puts("1");

}

else

{

lcd_gotoxy(15,1);

lcd_puts("0");

}

// print pressed key

lcd_gotoxy(8, 1);

lcd_puts("      ");

if (value >= 1016)

{

lcd_gotoxy(8, 1);

lcd_puts("None");		// None

}

else if (value == 0)

{

lcd_gotoxy(8, 1);

lcd_puts("Right");		// Right

}

else if (value == 101)

{

lcd_gotoxy(8, 1);

lcd_puts("Up");			// Up

}

else if (value == 245)

{

lcd_gotoxy(8, 1);

lcd_puts("Down");		// Down

}

else if (value == 402)

{

lcd_gotoxy(8, 1);

lcd_puts("Left");		// Left

}

else if (value == 650)

{

lcd_gotoxy(8, 1);

lcd_puts("Select");		// Select

}

}

  - Screenshot of SimulIDE circuit when "Power Circuit" is applied.

- UART. Submit:

  - (Hand-drawn) picture of UART signal when transmitting data DE2 in 4800 7O2 mode (7 data bits, odd parity, 2 stop bits, 4800 Bd),

  - Listing of code for calculating/displaying parity bit.

// Odd Parity bit counter

if(value %2 == 0)

{

lcd_gotoxy(15,1);

lcd_puts("1");

}

else

{

lcd_gotoxy(15,1);

lcd_puts("0");

}